This document says how to write the teimage.bin into AT's flashrom through LPT.. First of all, please make sure that there is AT45DB081*/ AT45DB161*/AT45DB321* chipset in your modem/router.The flash program is used for them ONLY. There is a 6-pin interface in modem/router used for connected to the LPT of computer. the connection of 6-pin interface to chipset 1---AT45DB161* 13 2---AT45DB161* 14 3---NOT connected to AT45DB161* (But connected to 4th-pin of LPT with invariable 3.3v voltage) 4---AT45DB161* 11 5---AT45DB161* 12 6---AT45DB161* 7 The connection of LPT to the 6-pin interface(make a connection cable following method) LPT 6-pin interface 4 -------- 3 5 -------- 1 6 -------- 5 7 -------- 4 11 -------- 2(WITH 100¦¸ Resistance 0.25w) 18 -------- 6 Make a cable in above method. PUT the teimage.bin into the same directory of loadfi.exe.(Suggest put them into a partition that the real-mode DOS can recognise,FAT16/FAT32 etc.) Reboot to real-mode DOS(NEEDED WIN98 bootdisk). DON'T Power up the modem/router , USE the cable Connect LPT to the 6-pin interface. Power up the modem/router, run the loadfi.exe, press "1" to write the teimage.bin into flashrom. P.S : IF the loadfi print a message "Cable1: No Connection", For this: 1. You maybe have to reverse the 6-pin order of cable. 2. Maybe there is a 2-pin interface have to be short. 3. Check the cable was correctly made....